ifm provides all-in-one control and monitoring for quarter-turn valves

Featuring compact construction and easy mounting, the new MVQ201 valve sensor from ifm electronic not only monitors and reports valve position to the nearest degree, but also has an integral M12 connector that can be used for connecting the valve. With this option, the valve is controlled via the same IO-Link connection used to monitor the sensor, which eliminates the need for separate valve wiring, thereby reducing both installation complexity and cost.

In use, the MVQ201 sensor continually monitors the valve position and makes the position data available to the control system via an IO-Link interface. The sensor can also be programmed to detect changes, such as increased opening and closing times, which might indicate valve faults or blockages, and to report these changes via IO-Link. This makes it an ideal choice for use in condition-based maintenance applications.

Conventional valve-open and valve-closed switching outputs are provided and can be set for normally open or normally closed operation. The valve positions corresponding to the open and closed states can be readily programmed with a convenient one-click auto-teach function. MVQ201 sensors also incorporate a clearly visible status LED that shows the valve position at a glance.

Suitable for use with valves having shafts up to 38 mm in diameter, MVQ201 valve position sensors have an IP65/IP67 protection rating and an operating temperature range of -25 to +70 ºC. A range of accessory mounting adaptors is available to further simplify installation.

=== END ===

(248 words)